AUTONOMOUS ROBOT

User Manual

By: Phenyo Diale

# 1. Introduction

The Autonomous Navigation Robot is designed to move independently while avoiding obstacles using ultrasonic sensors. It is powered by an ESP32 microcontroller that controls motor movement, LED indicators, and app-based communication for remote control. The robot can navigate forward, detect obstacles in its path, and adjust its direction accordingly, while LEDs provide real-time status feedback on its operation.

This project demonstrates the integration of mechanical, electrical, and programming systems into a single working platform, making it useful for learning, experimentation, or as a base for more advanced robotics applications.

⚠️ Safety Note: Operate the robot only on flat, smooth surfaces. Keep it away from water, dust, and extreme heat. Always double-check wiring connections before powering on to prevent damage to components. Disconnect the battery when not in use.

# 2. Package contents

1 × Autonomous Navigation Robot (Fully Assembled)
 Includes chassis, wheels, motors, sensors, motor driver, ESP32, and Arduino Nano BLE 33 Sense pre-installed.

1 × Rechargeable Battery Pack (4 × 18650 cells pre-installed)

1 × USB Cable (programming updates)

1 × Mobile App (download link / QR code)

1 × User Manual (Setup & Operation Guide)

# 3. Operating Instructions

- Turn ON power switch.

- Place robot on a flat surface.

- Robot moves forward until an obstacle is detected.

- If obstacle detected on left or right, robot turns in opposite direction.

- LEDs provide live status indicators.

- App can be used for manual control.

# 4. Modes of operations

The Autonomous Navigation Robot can operate in the following modes:

- Autonomous Mode (Default)

  - The robot uses its ultrasonic sensors to detect obstacles.

  - It automatically moves forward, stops when an obstacle is detected, and decides whether to turn left or right.

  - LED indicators display robot status:

🟢 Green - Moving forward

🟡 Yellow - Turning/obstacle detected

🔴 Red – Stopped

- Manual Remote Mode

  - Using the companion mobile app (via Wi-Fi), the user can directly control the robot’s movement: forward, backward, stop, left, and right.

  - Speed can be adjusted using PWM control through the app.

  - LED indicators still function to provide status feedback.

- Standby / Idle Mode

  - The robot remains powered but motors are off.

  - Red LED remains on to indicate standby.

# 5. Obstacle Avoidance

The robot is equipped with ultrasonic sensors that allow it to detect and avoid obstacles in its path. This feature ensures safe navigation and prevents collisions with surrounding objects.

How It Works

- The ultrasonic sensors continuously measure the distance between the robot and nearby objects.

- When an obstacle is detected within a certain range (e.g., 10–20 cm), the robot will automatically stop and change its direction.

- The robot resumes forward movement once the path is clear.

How to Use

- Turn on the robot and place it on a flat surface.

- Make sure the front of the robot faces an open space.

- Activate the Obstacle Avoidance Mode (via switch, app, or command – depending on your design).

- The robot will move forward and automatically avoid obstacles without user input.

Tips for Best Performance

- Use the robot indoors or on smooth surfaces.

- Keep sensor areas clean and free from dust.

- Avoid direct sunlight or highly reflective surfaces, as they may affect sensor readings.

Limitations

- Very small, thin, or transparent objects may not always be detected.

- Performance may vary on uneven or cluttered surfaces.

# 6. Controls

The robot can be controlled in two modes: Manual Control and Autonomous Obstacle Avoidance Mode.

A) Manual Control

- Power Switch – Turns the robot on and off.

- Forward Button – Moves the robot straight ahead.

- Backward Button – Moves the robot backward.

- Left Button – Turns the robot left.

- Right Button – Turns the robot right.

- Stop Button – Immediately stops all movement.

B) Autonomous Mode (Obstacle Avoidance)

- When enabled, the robot moves forward on its own.

- If the ultrasonic sensors detect an obstacle:

  - The robot stops.

  - It chooses a new direction (left or right).

  - It continues moving until the path is clear.

C) LED Indicators (if included)

- Green LED – Robot is powered on and ready.

- Blue LED – Manual control mode is active.

- Red LED – Obstacle detected, or robot stopped for safety.

# 6. Battery Management

This robot is powered by Li-ion 18650 rechargeable cells. These batteries provide reliable power, but they must be handled with care.

A) Installing the Batteries

- Make sure the robot is switched OFF.

- Open the battery compartment.

- Insert the 18650 cells, following the + (positive) and – (negative) polarity markings.

- Close the battery compartment securely.

B) Charging the Batteries

- This robot does not have an internal charging system.

- Remove the 18650 cells when they need charging.

- Use a dedicated external Li-ion 18650 charger.

- Place the cells in the charger according to polarity markings.

C) Battery Safety Tips

- Only use quality 18650 Li-ion cells with protection circuits.

- Do not mix old and new cells or cells of different brands.

- Never short-circuit the battery terminals.

- Do not expose to fire, water, or temperatures above 60 °C.

- If a battery swells, leaks, or gets very hot, do not reuse it.

D) Maximizing Battery Life

- Recharge the batteries before they are completely drained (below 3.0 V per cell).

- For storage longer than 1 month, keep the cells about 40–60% charged.

- Store batteries in a cool, dry place away from direct sunlight.

- Remove the batteries from the robot if not used for long periods.

E) Battery Replacement

- The robot is compatible with standard 18650 Li-ion rechargeable cells (3.7 V nominal, 4.2 V full charge).

- Replace only with cells of the same type and size.

- Always follow polarity markings when installing.

- Dispose of used batteries responsibly at recycling points

# 7. Maintenance

| Category | Tasks | Notes |
| --- | --- | --- |
| A) Cleaning | - Wipe robot with a soft, dry cloth. - Use slightly damp cloth for stubborn dirt (avoid soaking). - Keep ultrasonic sensors clean and dust-free. - Avoid strong chemicals or solvents. | Ensures accurate sensor readings and neat appearance. |
| B) Battery Care | - Remove Li-ion 18650 cells when robot is unused for long periods. - Inspect batteries for swelling, leaks, or damage. - Store in a cool, dry place away from sunlight. | Prevents battery damage and improves lifespan. |
| C) Motor & Wheels | - Check wheels for dust, hair, or debris - Tighten chassis screws regularly. | Keeps robot moving smoothly and prevents wear. |
| D) Electrical Connections | - Inspect jumper wires and solder joints. - Do not pull wires by the cable, use connectors. - If a sensor/LED fails, check wiring first. | Maintains reliable electrical performance. |
| E) Software & Updates | - Back up ESP32/Arduino code. - Update software only with verified versions. - Test motors and sensors after updates. | Prevents software-related malfunctions. |

# 8. Troubleshooting

| Problem | Possible Cause | Solution |
| --- | --- | --- |
| Robot does not turn on | - Batteries are dead or incorrectly installed- Power switch off | - Replace or charge batteries- Ensure correct polarity- Turn power switch on |
| Robot stops moving suddenly | - Battery voltage too low- Obstacle detected | - Recharge or replace batteries- Remove obstacles from the path |
| Motors not working | - Loose motor wires- Motor failure | - Check and reconnect wires- Replace motor if needed |
| Ultrasonic sensors not detecting obstacles | - Sensors dirty or blocked- Wiring loose- Sensor failure | - Clean sensor surface- Check connections- Replace sensor if damaged |
| Robot moves in wrong direction | - Wiring or motor connections reversed- Software issue | - Check motor wiring polarity- Re-upload verified code |
| LEDs not lighting | - Loose wires- Burnt LED- Code issue | - Check connections- Replace LED if needed- Verify software controls |
| App or remote not responding | - Bluetooth/Wi-Fi not connected- App malfunction | - Ensure robot and app are paired- Restart app and robot- Update software |
| Robot overheating. | - Prolonged use - Motor or battery strain | - Turn off and let it cool - Check motors for debris - Ensure batteries are healthy |

Quick fix guide

| Issue | Fast Solution |
| --- | --- |
| Robot won’t turn on | Check battery installation and polarity. Replace or recharge 18650 cells. Turn the power switch on. |
| Robot stops moving | Remove any obstacles. Check battery voltage; recharge if low. |
| Ultrasonic sensors not detecting | Clean sensor surface. Ensure wiring is secure. |
| Robot moves in wrong direction | Check motor wiring polarity. Re-upload verified code if needed. |
| LEDs not lighting | Check connections. Replace LED if damaged. Verify software commands. |

# 9. Technical Specifications

Component Placement

- Front of Robot: Front ultrasonic sensor, green LED indicator.

- Top of Chassis: ESP32 board mounted securely.

- Wheels & Motors: Motors attached to chassis corners, connected via motor driver.

- Back / Side: Second ultrasonic sensor for side/back detection.

- Battery Compartment: Under chassis, removable 18650 cells.

- Power Switch: Accessible on top or side for easy ON/OFF control.

- LEDs: Mounted where visible to indicate status.